Comparative Analysis of ECG Preprocessing Methods for Deep Learning-Based Arrhythmia Detection Using CNN-LSTM

Reference

- Patient-adaptive and clinically interpretable arrhythmia detection from ECG using lightweight hybrid deep learning

https://doi.org/10.1007/s13534-026-00615-7

- Enhanced Arrhythmia Detection Employing CNNLSTM Hybrid Architectures and Advanced Signal

Processing Techniques

https://doi.org/10.1007/s44196-026-01510-w

DATABASE

- https://www.physionet.org/content/mitdb/1.0.0/

TUJUAN

- Membandingkan preprocessing dan melihat efeknya terhadap detection

METHOD

- Independent : ECG Preprocesssing Method (Diubah)

| Scenario | Filtering | Normalization | Sampling rate | Purpose |
| --- | --- | --- | --- | --- |
| 1 | Raw Signal | - | 360 Hz | Evaluates the baseline performance of the CNN-LSTM model without any preprocessing |
| 2 | Butterworth Bandpass (0.5 – 45 Hz) | - | 360 Hz | Measures conventional high/low-frequency and powerline noise removal |
| 3 | DWT Symlet-10 (sym10) | - | 360 Hz | Compares Wavelet-based baseline wander removal against Butterworth filtering while preserving waveform morphology |
| 4 | Best filtering method (2 or 3) | Min-Max Scaling ([-1,1]) | 360 Hz | Assesses the effect of amplitude range mapping on model optimization and convergence |
| 5 | Best filtering method | Z-score Standardization (μ = 0, σ = 1) | 360 Hz | Compares distributional scaling stability (Z-Score) against boundary scaling (Min-Max) |
| 6 | Best filtering method | Best normalization method | 100 Hz | Tests computational efficiency, memory usage, and accuracy at a moderate sampling frequency |
| 7 | Best filtering method | Best normalization method | 50 Hz | Tests the lower-bound tolerance of sampling rate reduction on model performance |

- Dependent : Model Performance (Diukur)

|  | Accuracy (%) | Precision (%) | Recall (%) | F1-Score (%) | Specificity (%) | Inference Latency (%) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |

- Control : Dataset utama, Data split, Patient independence, ECG lead, Sampling rate, Segmentation, R-peak, Normalization, Optimizer, Learning rate, Batch size, Epoch, Loss function, Random seed, Class balancing